Optimal TCP NewReno Socket Settings for Short Transfers in Home, Mobile and Long-Distance Networks using ns-3

Yi Chen

Department of Engineering

University West

Trollhättan, Sweden

yi.chen@student.hv.se

Abstract—The time needed to download a short file depends not only on the network path but also on the TCP settings of the connection. This paper uses the ns-3 network simulator to find which combination of three TCP NewReno socket parameters, the initial congestion window, the segment size and the delayed acknowledgment count, gives the shortest transfer time for a 1 MB file in three scenarios: a home connection, a mobile connection and a long-distance connection from Sweden to Australia. The link speeds and delays were measured by the author, and the loss probabilities were selected using published measurements as a reference. All 27 combinations were simulated in each scenario, with 30 independent runs where the link has random loss, giving 1647 runs. The same setting was fastest in all three scenarios: an initial window of 10 segments, 1448-byte segments and an acknowledgment for every segment, with mean transfer times of 0.21 s, 0.25 s and 2.71 s. The slowest setting took 3.7 to 5.4 times longer. The delayed acknowledgment count had the largest effect, mainly because in the ns-3 NewReno model it slows the growth of the sending window.

Keywords—TCP NewReno, transfer time, initial congestion window, delayed acknowledgment, segment size, ns-3, simulation

I. Introduction

How fast a file is downloaded depends on the network path, but also on how the Transmission Control Protocol (TCP) connection is configured. The author wanted to find out how much the TCP configuration affects the transfer speed. This paper compares three socket settings of TCP NewReno: the initial congestion window, the segment size, and the number of segments the receiver waits for before it sends an acknowledgment (ACK). For a short transfer such as a 1 MB file, these settings matter because the connection finishes within a few round trips, while its sending window is still growing from a small initial value [1], [2].

The author measured the round-trip time (RTT) from home in Gothenburg, Sweden: 20 ms to a Swedish server (www.kth.se), 32 ms to the same server over a 5G mobile phone hotspot, and 360 ms to a server in Australia (mirror.aarnet.edu.au). The ns-3 simulator uses one set of default values for these settings, but a setting that suits a short path may not suit a long or lossy one. For example, waiting for several segments before acknowledging saves ACK packets, but on a long path every extra round trip costs a third of a second (RTT to Australia: 360 ms ≈ 1/3 s).

This paper therefore asks: does the combination of TCP NewReno settings that gives the shortest transfer time differ between a home, a mobile and a long-distance scenario? To answer it, three values of each setting were combined in all 27 ways and simulated in ns-3 for each scenario, and the congestion window was traced to see why some combinations are slower than others.

II. Background

A. Slow Start and the Initial Window

Jacobson introduced slow start so that a new connection would not send a full window into the network at once [1]. The sender starts with a small congestion window (cwnd), 1 segment in the original algorithm, and increases it by 1 segment for every ACK of new data. When every segment is acknowledged, the window doubles in each RTT, so reaching a window of W segments takes about log2 W round trips.

Dukkipati et al. measured the effect of raising the initial window from 3 to 10 segments on Google's servers. The average latency of web responses improved by about 10%, and the largest gains were seen on paths with a high RTT [2]. An initial window of 10 segments was later described in RFC 6928 [3], and it is the default value in ns-3.

B. Delayed Acknowledgments

To reduce the number of small packets, a receiver can delay its ACK until a number of segments have arrived or a timer expires. With fewer ACKs, a sender that grows its window per ACK grows more slowly: with one ACK for every second segment, the window grows by a factor of about 1.5 per RTT instead of 2 [2], [4]. RFC 3465 proposes Appropriate Byte Counting (ABC), in which the window grows with the number of bytes acknowledged rather than the number of ACKs, to remove this penalty [4].

C. Segment Size

The congestion window is counted in segments, so a larger segment carries more bytes per round trip for the same window. With smaller segments, the initial window therefore holds fewer bytes, so the window growth starts from a lower point: 10 segments of 536 bytes are about 5 kB, compared with about 14 kB for 10 segments of 1448 bytes.

D. Simple Simulation Models

Floyd and Paxson argue that the Internet is too varied to simulate completely, and that for a focused question the simplest scenario that shows the mechanism is often the best choice, provided that conclusions are not drawn from a single scenario and that their robustness is checked when the model conditions change [5]. Floyd and Kohler add that a model should be specific to the research question, that modeling should go hand in hand with measurement, and that researchers need to know when a model might lead to wrong results [6].

III. Method

A. Simulation Program

The simulations use ns-3 version 3.48 [7]. The program extends the tutorial example fifth.cc from the course laboratory work: two nodes are connected by one point-to-point link that carries a single TCP flow. The main changes are the following. The link speed, the one-way delay, the loss rate and the three TCP parameters are command-line options, so one program covers all scenarios. TCP NewReno is chosen, because CUBIC is the default in ns-3. The sender uses BulkSendApplication, as in the ns-3 example tcp-linux-reno.cc [8], to send exactly 1 MB (106 bytes), and the receiver records the time at which the last byte arrives. The transfer time is measured from the start of the sender application, so it includes the connection handshake. The send and receive buffers are 4 MB in all runs, so that the receiver window never limits the sender. Random loss uses a rate error model with a packet loss probability, the same measure that measurement studies report. The program, the run script, the results and the plotting script are available on GitHub [9].

B. Scenarios

Table I lists the three scenarios. The speeds and delays were measured by the author with the same laptop on 18 October 2026. The one-way delay is half of the average RTT of 10 pings: to www.kth.se over the home Wi-Fi (2.4 GHz band) and over a 5G mobile phone hotspot, and to mirror.aarnet.edu.au in Australia over the home Wi-Fi. The home Wi-Fi was slower than the 5G hotspot (63 vs. 160 Mbps). The web sites of Australian universities that the author tried were served from European servers through content delivery networks, so a mirror server located in Australia was used instead. The link speeds are the download speeds measured with the Bredbandskollen speed test, and the long-distance scenario uses the home connection and therefore its speed. The loss probabilities were not measured but selected as model values. The mobile value of 0.01% equals the median TCP packet loss rate measured in a commercial mmWave 5G network in the US [10]. The long-distance value of 0.015% was chosen within the lowest loss class, below 0.1%, that long-term PingER measurements report for Europe and Australasia [11]; the source gives no exact value for this path. The short home path has no random loss.

TABLE I.

Network Scenarios

| Scenario | Link speed | Average RTT | One-way delay | Packet loss |
| --- | --- | --- | --- | --- |
| Home | 63 Mbps | 20 ms | 10 ms | 0 |
| Mobile (5G) | 160 Mbps | 32 ms | 16 ms | 0.01% |
| Long distance | 63 Mbps | 360 ms | 180 ms | 0.015% |

C. Parameters and Runs

Table II shows the three parameters and the values tested. The largest segment size, 1448 bytes, is the payload that fits in a 1500-byte Ethernet frame with the TCP timestamp option. Every combination of values was simulated in every scenario, which gives 3 × 3 × 3 = 27 combinations per scenario. In the text, the delayed ACK count is called DelAckCount, as in ns-3.

TABLE II.

TCP Parameters Tested

| Parameter | Meaning | Values | ns-3 default |
| --- | --- | --- | --- |
| InitialCwnd | Segments sent before the first ACK | 1, 4, 10 | 10 |
| SegmentSize | Data bytes per segment | 536, 1000, 1448 | 536 |
| DelAckCount | Segments received per ACK | 1, 2, 4 | 2 |

Parameter names and default values are those of the ns-3 class TcpSocket, defined in the source file tcp-socket.cc of ns-3.48 [12].

The mobile and long-distance scenarios have random loss, so each combination was run 30 times with different random streams (RngRun 1 to 30, seed 1), and the author used the mean. The home scenario has no random loss and every run gives the same result, so each combination was run once. In total this gives 27 + 810 + 810 = 1647 runs.

To explain the results, the author traced the congestion window in the long-distance scenario without random loss, for the best setting and for the same setting with one parameter changed.

IV. Results

A. Best Setting in Each Scenario

Table III shows the fastest and the slowest setting in each scenario. The same setting was the fastest in all three scenarios: an initial window of 10 segments, 1448-byte segments and an ACK for every segment, written (10, 1448, 1). The slowest was also the same, (1, 536, 4). The scenarios differ in how much slower the worst setting is: 4.5 times at home, 5.4 times in the mobile scenario and 3.7 times over the long distance.

TABLE III.

Fastest and Slowest Setting per Scenario

| Scenario | Fastest (10, 1448, 1) | Slowest (1, 536, 4) | Second fastest | Slowest / fastest |
| --- | --- | --- | --- | --- |
| Home | 0.214 s | 0.967 s | (10, 1000, 1) 0.224 s | 4.5 |
| Mobile | 0.245 s | 1.313 s | (10, 1000, 1) 0.262 s | 5.4 |
| Long distance | 2.713 s | 10.148 s | (10, 1000, 1) 2.773 s | 3.7 |

Settings are written (InitialCwnd, SegmentSize, DelAckCount). Mobile and long distance: mean of 30 runs. Home: one run.

With the selected loss probabilities, random loss had little effect. Only 55 of the 810 mobile runs and 64 of the 810 long-distance runs were slower than the loss-free run of the same setting, and the 30 runs of the best setting differed by less than 0.01 s in both scenarios. The differences between the settings are therefore mainly set by the RTT and the three parameters.

B. Effect of Each Parameter

Fig. 1 shows the mean transfer time of the best setting and of the same setting with one parameter changed to its least favorable value. DelAckCount has the largest effect in all three scenarios: with DelAckCount = 4 the transfer takes 1.4 times longer at home, 1.9 times longer in the mobile scenario and 1.8 times longer over the long distance. The initial window has the second largest effect: an initial window of 1 segment adds 30% at home, 43% in the mobile scenario and 41% over the long distance. 536-byte segments add 15% to 21%.

Fig. 1.  Mean transfer time of the best setting (blue) and of the same setting with one parameter changed (gray) in each scenario. Mobile and long distance: mean of 30 runs. Home: one run.

C. Why the Parameters Matter: Window Growth

Table IV shows the congestion window in each round trip in the long-distance scenario without random loss. The initial window and the segment size change only the starting point. With a smaller initial window (4 or 1 segment) or smaller segments (1000 or 536 bytes), the window starts smaller but still doubles in each round trip, so the transfer needs up to three extra round trips, as the slow-start latency model of Dukkipati et al. predicts [2]. DelAckCount changes the growth rate instead. In the ns-3 NewReno model the window grows by 1 segment per ACK during slow start, however many segments the ACK covers [7]. With DelAckCount = 2 the window therefore grows by a factor of 1.5 per round trip, and with DelAckCount = 4 by a factor of 1.25. The gap to the best setting widens in every round trip, which is why this parameter has the largest effect.

TABLE IV.

Congestion Window (kB) in Each Round Trip, Long Distance, No Random Loss

| Setting | 1 | 2 | 3 | 4 | 5 | 6 | 7 | Growth per RTT (approx.) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Best (10, 1448, 1) | 14 | 29 | 58 | 116 | 232 | 463 | 927 | × 2 |
| InitialCwnd = 4 | 6 | 12 | 23 | 46 | 93 | 185 | 371 | × 2 |
| InitialCwnd = 1 | 1 | 3 | 6 | 12 | 23 | 46 | 93 | × 2 |
| SegmentSize = 1000 | 10 | 20 | 40 | 80 | 160 | 320 | 640 | × 2 |
| SegmentSize = 536 | 5 | 11 | 21 | 43 | 86 | 172 | 343 | × 2 |
| DelAckCount = 2 | 14 | 22 | 33 | 49 | 74 | 110 | 165 | × 1.5 |
| DelAckCount = 4 | 14 | 19 | 25 | 30 | 39 | 48 | 59 | × 1.25 |

One parameter is changed from the best setting (10, 1448, 1). Columns 1 to 7 are the round trips. The window is read in the middle of each RTT of 0.36 s, so column 1 is the initial window.

D. Estimating the Transfer Time from the Number of Rounds

The large effect of DelAckCount was unexpected: before the simulations, the author had expected the segment size to matter most. In the long-distance scenario without random loss, a simple count of round trips shows where the effect comes from. The 1 MB file needs 1 000 000 ÷ 1448 ≈ 691 segments, RTT = 0.36 s, and the ns-3 NewReno window grows by 1 segment per ACK. A transfer that needs n rounds takes one RTT for the handshake, one RTT per round except the last, and half an RTT for the last data to arrive:

T ≈ RTT + (n − 1) · RTT + RTT/2 = (n + 0.5) · RTT	(1)

1) (10, 1448, 1): Every segment is acknowledged, so the window doubles each round (Table V). With n = 7, (1) gives 7.5 × 0.36 s ≈ 2.70 s; the simulated time is 2.71 s.

TABLE V.

Estimated Segments Sent per Round, (10, 1448, 1), Long Distance

| Round | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Est. segments sent | 10 | 20 | 40 | 80 | 160 | 320 | 61 |
| Est. total sent | 10 | 30 | 70 | 150 | 310 | 630 | 691 |
| Window (kB) | 14 | 29 | 58 | 116 | 232 | 463 | 927 |

Estimated from the window, which doubles each round; round 7 sends the last 61.

2) (10, 1448, 2): One ACK per 2 segments, so the window grows by half each round, a factor of 1.5 (Table VI). With n = 9, (1) gives 9.5 × 0.36 s ≈ 3.42 s; the simulated time is 3.46 s. The two extra rounds, 2 × 0.36 s = 0.72 s, explain almost all of the 0.74 s difference between the two settings. The estimate ignores transmission time, so it does not hold for the short home and mobile paths.

TABLE VI.

Estimated Segments Sent per Round, (10, 1448, 2), Long Distance

| Round | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Est. segments sent | 10 | 15 | 23 | 34 | 51 | 76 | 114 | 171 | 197 |
| Est. total sent | 10 | 25 | 48 | 82 | 133 | 209 | 323 | 494 | 691 |
| Window (kB) | 14 | 22 | 33 | 49 | 74 | 110 | 165 | 248 | 371 |

Estimated from the cwnd-trace window (≈ × 1.5); round 9 sends the last 197.

V. Discussion

A. Delayed ACKs: a Property of the Implementation

The growth factor of 1.5 per round trip with DelAckCount = 2 is the value that RFC 3465 gives for a sender that grows its window per ACK [4]. The same RFC proposes byte counting to remove this effect: with byte counting, each ACK would add as many segments as it covers. The large effect of DelAckCount in Fig. 1 therefore depends on how the ns-3 NewReno model counts ACKs, and an implementation that counts bytes may give a different result. The best value, an ACK for every segment, also doubles the number of ACK packets, but this cost is not measured here.

B. Limitations

The model is a simple scenario of the kind that Floyd and Kohler discuss [6]. Each scenario is a single link described by three numbers. There is no competing traffic, so the only queueing comes from the flow's own packets at the sender. The RTT does not vary, and the losses are independent rather than in bursts. The speeds and delays were measured on one morning and may differ at busier times. The home speed was measured over 2.4 GHz Wi-Fi, which is likely slower than the broadband connection itself. The loss probabilities are model values based on studies of other networks [10], [11]. Only one file size and one TCP variant were tested. The results show how each setting affects the transfer time in a controlled model, not the transfer times a user would see on a real network.

VI. Conclusion

For the three scenarios and parameter values tested, (10, 1448, 1) gave the shortest transfer time at home, over a mobile link and to Australia, so the best choice did not depend on the scenario. In ns-3, DelAckCount had the largest effect: a value of 4 made transfers up to 1.9 times slower. The default initial window of 10 segments was already the best of the values tested. The size of the DelAckCount effect depends on how the implementation counts ACKs, so future work could repeat the study with byte counting or with TCP CUBIC, and with competing traffic on the link.

References

[1]	V. Jacobson, “Congestion avoidance and control,” ACM SIGCOMM Comput. Commun. Rev., vol. 18, no. 4, pp. 314–329, Aug. 1988.

[2]	N. Dukkipati et al., “An argument for increasing TCP’s initial congestion window,” ACM SIGCOMM Comput. Commun. Rev., vol. 40, no. 3, pp. 27–33, Jul. 2010.

[3]	J. Chu, N. Dukkipati, Y. Cheng, and M. Mathis, “Increasing TCP’s initial window,” RFC 6928, IETF, Apr. 2013.

[4]	M. Allman, “TCP congestion control with appropriate byte counting (ABC),” RFC 3465, IETF, Feb. 2003.

[5]	S. Floyd and V. Paxson, “Difficulties in simulating the Internet,” IEEE/ACM Trans. Netw., vol. 9, no. 4, pp. 392–403, Aug. 2001.

[6]	S. Floyd and E. Kohler, “Internet research needs better models,” ACM SIGCOMM Comput. Commun. Rev., vol. 33, no. 1, pp. 29–34, Jan. 2003.

[7]	ns-3 Project, “TCP models in ns-3,” ns-3 Model Library. [Online]. Available: https://www.nsnam.org/docs/release/3.48/models/html/tcp.html (accessed Oct. 18, 2026).

[8]	A. Bhargava, “tcp-linux-reno.cc,” ns-3.48 example program, examples/tcp. [Online]. Available: https://gitlab.com/nsnam/ns-3-dev/-/blob/ns-3.48/examples/tcp/tcp-linux-reno.cc (accessed Oct. 25, 2026).

[9]	Y. Chen, “DTA400 simulation project,” GitHub repository, 2026. [Online]. Available: https://github.com/Olivialinden/DTA400_simulation_project (accessed Oct. 18, 2026).

[10]	A. Narayanan et al., “A first look at commercial 5G performance on smartphones,” in Proc. Web Conf. (WWW ’20), Taipei, Taiwan, Apr. 2020, pp. 894–905, doi: 10.1145/3366423.3380169.

[11]	R. L. Cottrell, “PingER end to end Internet measurements: What we learn,” presented at OARC TechDay, San Francisco, CA, USA, Mar. 2011. [Online]. Available: https://indico.dns-oarc.net/event/15/contributions/151/attachments/162/349/PingER-LesCottrel.pdf (accessed Oct. 18, 2026).

[12]	ns-3 Project, “tcp-socket.cc,” ns-3.48 source code, src/internet/model. [Online]. Available: https://gitlab.com/nsnam/ns-3-dev/-/blob/ns-3.48/src/internet/model/tcp-socket.cc (accessed Oct. 18, 2026).